7x60w 4in1 Bee Eye

PLEASE READ THIS CAREFULLY BEFORE USE

# Skill parameters

Light Sources: 7x60W RGBW Original LED

Power Voltage: AC 100-240V, 50/60Hz, 500w

Single LED power: 60W

Color: RGBW automatic correction 16.7 million pixels

Electronic dimmer:0～100% linear

Control

DMX Channel: 23/35/51 Channel

Control Modes: DMX512

Software Upgrade: Firmware update via DMX link

Pan/Tilt

Pan/Tilt: 540°

Construction

Display: LCD Display

Data In/Out socket: 3-pin XLR sockets Power Socket: Powercon in/out

Protection Rating: IP20

Features

Motorized liner zoom system,

zoom range: 4.5°~45° Outstanding color macro effect,CTO,Kaleido effects,led single controlled

Dimmer: 0~100% smooth dimming Variable strobe effects

Fast, quiet operation and flicker free

/ 270° Pan/Tilt Resolution: 16bit

| 23 | reset | 0-199 | invalid |
| --- | --- | --- | --- |
|  |  | 200-205 | Reset after 5 seconds |
|  |  | 206-255 | invalid |
| 35 CH | Channel | DMX Value | Function |
| 1 | red | 0-255 | Red dimmer |
| 2 | Red fine | 0-255 | Red dimmer fine |
| 3 | green | 0-255 | Green dimmer |
| 4 | Green fine | 0-255 | Green dimmer fine |
| 5 | blue | 0-255 | Blue dimmer |
| 6 | Blue fine | 0-255 | Blue dimmer fine |
| 7 | white | 0-255 | White dimmer |
| 8 | White fine | 0-255 | White dimmer fine |
| 9 | CT | 0-255 | Linear CT control |
| 10 | Color Macro | 0-255 | Color Macro function |
| 11 | strobe | 0-3 | off |
|  |  | 4-103 | Synchronous strobe slow-fast （1HZ-25HZ） |
|  |  | 104-107 | on |
|  |  | 108-207 | Divide strobe from slow-fast （1HZ-25HZ） |
|  |  | 208-212 | on |
|  |  | 213-225 | Random strobe in low speed |
|  |  | 226-238 | Random strobe in middle speed |
|  |  | 239-251 | Random strobe in high speed |
|  |  | 252-255 | on |
| 12 | dimmer | 0-255 | Linear dimmer from dark-bright |
| 13 | Dimmer fine | 0-255 | Dimmer fine |
| 14 | pan | 0-255 | pan |
| 15 | Pan fine | 0-255 | Pan fine |
| 16 | tilt | 0-255 | tilt |
| 17 | Tilt fine | 0-255 | Tilt fine |
| 18 | function | 0-255 |  |

| 19 | reset | 0-199 | invalid |
| --- | --- | --- | --- |
|  |  | 200-205 | Reset after 5 seconds |
|  |  | 206-255 | invalid |
| 20 | zoom | 0-255 | zoom |
| 21 | Lens rotating | 0-127 | 0~60 degree |
|  |  | 128-191 | Forward limitless rotate |
|  |  | 192-255 | Reverse limitless rotate |
| 22 | pattern | 0-255 | Pattern rotate |
| 23 | Pattern speed | 0-255 | Pattern speed |
| 24 | Pattern effect fade | 0-255 |  |
| 25 | Pattern effect R | 0-255 |  |
| 26 | Pattern effect G | 0-255 |  |
| 27 | Pattern effect B | 0-255 |  |
| 28 | Pattern effect W | 0-255 |  |
| 29 | Pattern dimmer | 0-255 | Pattern 0-100% dimmer |
| 30 | Background dimmer | 0-255 | Background 0-100% dimmer |
| 31 | Pattern transition | 0-255 |  |
| 32 | Pattern angle | 0-255 | Pattern design |
| 33 | Prospect strobe | 0-255 | Prospect strobe（with 11 chs） |
| 34 | Background strobe | 0-255 | Background strobe（with 11 chs） |
| 35 | Background selection | 0-255 |  |
| 51 CH | Channel | DMX Value | Function |
| 1 | X | 0-255 | 0-540 degree |
| 2 | X fine | 0-255 | 16 bit |
| 3 | Y | 0-255 | 0-205 degree |
| 4 | Y fine | 0-255 | 16 bit |
| 5 | XY speed | 0-255 | Fast-slow |
| 6 | zoom | 0-255 | Small-big |
| 7 | rotating | 0-127 | 0~60 degree |
|  |  | 128-191 | Forward fast-slow |

| 7 | rotating | 200-255 | Reverse slow-fast |
| --- | --- | --- | --- |
| 8 | dimmer | 0-255 | Dark-bright |
| 9 | strobe | 0-3 | on |
|  |  | 4-200 | Synchronous strobe slow-fast |
|  |  | 201-215 | Random strobe in low speed |
|  |  | 216-234 | Random strobe in midlle speed |
|  |  | 235-255 | Random strobe in high speed |
| 10 | R dimmer | 0-255 |  |
| 11 | G dimmer | 0-255 |  |
| 12 | B dimmer | 0-255 |  |
| 13 | W dimmer | 0-255 |  |
| 14 | CT | 0-255 | Linear CT control |
| 15 | Wash | 0-255 |  |
| 16 | Static effect | 0-255 | 5 no. One effect |
| 17 | Dynamic effect | 0-255 | 5 no. One effect |
| 18 | Dynamic effect speed | 0-127 | Forward fast-slow |
|  |  | 128-255 | Reverse slow-fast |
| 19 | Background R | 0-255 | Dark-bright |
| 20 | Background G | 0-255 | Dark-bright |
| 21 | Background B | 0-255 | Dark-bright |
| 22 | Background W | 0-255 | Dark-bright |
| 23 | reset | 0-199 | invalid |
|  |  | 200-205 | Reset after 5s |
|  |  | 206-255 | invalid |
| 24 | R1 LED dimmer | 0-255 | R1 LED dimmer |
| 25 | G1 LED dimmer | 0-255 | G1 LED dimmer |
| 26 | B1 LED dimmer | 0-255 | B1 LED dimmer |
| 27 | W1 LED dimmer | 0-255 | W1 LED dimmer |
| … | … | …… | …… |
| 48 | R7 LED dimmer | 0-255 | R7 LED dimmer |
| 49 | G7 LED dimmer | 0-255 | G7 LED dimmer |
| 50 | B7 LED dimmer | 0-255 | B7 LED dimmer |
| 51 | W7LED dimmer | 0-255 | W7 LED dimmer |

Channels List

| 23 CH | Channel | DMX Value | Function |
| --- | --- | --- | --- |
| 1 | X | 0-255 | 0-540 degree |
| 2 | X fine | 0-255 | 16 bit |
| 3 | Y | 0-255 | 0-205 degree |
| 4 | Y fine | 0-255 | 16 bit |
| 5 | XY speed | 0-255 | Fast-slow |
| 6 | zoom | 0-255 | Angle from small-big |
| 7 | rotating | 0-127 | 0~60 degree |
|  |  | 128-191 | Antiwise fast-slow |
|  |  | 200-255 | Forward slow-fast |
| 8 | dimmer | 0-255 | From dark-bright |
| 9 | strobe | 0-3 | on |
|  |  | 4-200 | Synchronous strobe from slow-fast |
|  |  | 201-215 | Random strobe in low speed |
|  |  | 216-234 | Random strobe in middle speed |
|  |  | 235-255 | Random strobe in high speed |
| 10 | R dimmer | 0-255 | From dark-bright |
| 11 | G dimmer | 0-255 | From dark-bright |
| 12 | B dimmer | 0-255 | From dark-bright |
| 13 | W dimmer | 0-255 | From dark-bright |
| 14 | CT | 0-255 | Linear CT control |
| 15 | wash | 0-255 | Inner color gradual change |
| 16 | Static effect | 0-255 | 5 no. One effect |
| 17 | Dynamic effect | 0-255 | 5 no. One effect |
| 18 | Dynamic effect speed | 0-127 | Forward from fast-slow |
|  |  | 128-255 | Reverse from slow-fast |
| 19 | Background R | 0-255 | Dark-bright |
| 20 | Background G | 0-255 | Dark-bright |
| 21 | Background B | 0-255 | Dark-bright |
| 22 | Background W | 0-255 | Dark-bright |