Specification of projection lamp

First of all, thank you for using our products. It uses the most advanced light source in the world - cold LED as the light source, which overcomes the shortcomings of traditional light sources, such as high power consumption, short service life and high temperature. It has the characteristics of low power consumption, long service life (generally up to about 60000 hours), high brightness and bright colors.It is an ideal product for stage lighting and various effect decoration.

1、 Operation instructions of display board

After starting up, press the menu key (menu key) to switch the menu, press the up key or down key to select the set value. After setting, press the Enter key to save and the display will flash to indicate that the data is saved.

Menu up and down confirmation

Basic usage:

Menu key: select various functions

Up: function parameters are selected step by step

Down: function parameter decreasing selection

Confirm (): confirm the current menuENTER

2、 Menu function

After power on, press the menu key (menu key) to successively display the following functions, and then press the OK key to confirm.(this sequence is the sequence of the new machine. If the lamp has used other modes and saved this mode before shutdown, the mode before the last shutdown will be displayed after starting up again). After setting other functions, you also need to press the save key to save, otherwise the effect of setting before shutdown will not appear after starting up next time

| A001 | 8In channel mode, press up / down to select the address code (A001 ~ A512), and press enter to save the address code |  |  |
| --- | --- | --- | --- |
| L001 | 4In channel mode, press up / down to select the address code (D001 ~ D512), and press enter to save the address code |  |  |
| d001 | 2In channel mode, press up / down to select the address code (D001 ~ D512), and press enter to save the address code |  |  |
| M000 | M000 | Dimming |  |
|  | M001 | Not bright |  |
|  | M002 | Light bead R is on |  |
|  | M003 | Light bead G is on |  |
|  | M004 | Light bead, G, R |  |
|  | M005 | Light bead B is on |  |
|  | M006 | Light beads R and B are on |  |
|  | M007 | Light beads g and B are on |  |
|  | M008 | Light beads R, G, B, on |  |
|  | M009 | Light bead w on |  |
|  | M010 | Light beads R and W are on |  |
|  | M011 | Light beads g and W are on |  |
|  | M012 | Light beads R, G and W are on |  |
|  | M013 | Light beads B and W are on |  |
|  | M014 | Light beads R, B and W are on |  |
|  | M015 | Light beads g, B and W are on |  |
|  | M016 | Light beads R, G, B and W are on |  |
|  | M017 | Gradual change |  |
|  | M018 | Pulse change |  |
|  | M019 | Lamp bead R → g → B → w circulating light |  |
|  | M020 | Lamp bead w → B → G-R cycle lamp |  |
|  | M021 | Lamp beads R, G, and B, w cycle light |  |
|  | M022 | Lamp beads R, W and G, B cycle running lamp |  |
|  | M023 | Lamp beads R, B, and G, W, cycle lamp |  |
|  | M024 | Lamp beads R, G to g, B to B, w to W, R cycle light |  |
|  | M025 | Lamp beads R, G, B to g, B, w to B, W, R, cycle light |  |
|  | M026 | Lamp bead r-g-b-w circulation |  |
|  | M027 | Voice control |  |
| S000 |  | S255 | For stroboscopic speed adjustment, press up / down to adjust between s000-s255Press enter to save the speed. |
| P000 |  | P255 | Function mode self walking speed (M000 mode), press the up key / down key to adjust between r000-r255.Press enter to save the speed. |
| R001 |  | R255 | Manually r brightness, when there is no console, press the up / down key to adjust the brightness between r000-r255, and press the Enter key to save the brightness. |
| G001 |  | G255 | Manually g brightness, when there is no console, press the up / down key to adjust the brightness between g000-g255, and press the Enter key to save the brightness. |
| B001 |  | B255 | Manually B brightness, when there is no console, press the up / down key to adjust the brightness between b000-b255, and press the Enter key to save the brightness. |
| W001 |  | W255 | Manually adjust the brightness. When there is no console, press the up / down key to adjust the brightness between w000-w255, and press the Enter key to save the brightness. |

3、 A001 and L001 D001 channel value description

A001 eight channel description

| passageway | Channel value | function |
| --- | --- | --- |
| 1 | 000-255 | Total dimming, linear dimming, from dark to bright |
| 2 | 000-255 | Total stroboscopic, from slow to fast |
| 3 | 0-29 | Dimming |
|  | 30-34 | Not bright |
|  | 35-39 | Light bead R is on |
|  | 40-44 | Light bead G is on |
|  | 45-49 | Light beads R and G are on |
|  | 50-54 | Light bead B is on |
|  | 55-59 | Light beads R and B are on |
|  | 60-64 | Light beads g and B are on |
|  | 65-69 | Light beads R, G, B, on |
|  | 70-74 | Light bead w on |
|  | 76-79 | Light beads R and W are on |
|  | 80-84 | Light beads g and W are on |
|  | 85-89 | Light beads R, G and W are on |
|  | 90-94 | Light beads B and W are on |
|  | 95-99 | Light beads R, B and W are on |
|  | 100-104 | Light beads g, B and W are on |
|  | 115-109 | Light beads R, G, B and W are on |
|  | 110-139 | Gradual change |
|  | 140-169 | Pulse change |
|  | 170-179 | Lamp bead R → g → B → w circulating light |
|  | 180-189 | Lamp bead w → B → G-R cycle lamp |
|  | 190-199 | Lamp beads R, G, and B, w cycle light |
|  | 200-209 | Lamp beads R, W and G, B cycle running lamp |
|  | 210-219 | Lamp beads R, B, and G, W, cycle lamp |
|  | 220-229 | Lamp beads R, G to g, B to B, w to W, R cycle light |
|  | 230-239 | Lamp beads R, G, B to g, B, w to B, W, R, cycle light |
|  | 240-249 | Lamp bead r-g-b-w circulation |
|  | 250-255 | Voice control |
| 4 | 000-255 | Channel 3 function speed, from slow to fast |
| 5 | 000-255 | R linear dimming, adjusting from dark to light. |
| 6 | 000-255 | G linear dimming, adjusting from dark to light. |
| 7 | 000-255 | B linear dimming, adjusting from dark to light. |
| 8 | 000-255 | W linear dimming, adjustable from dark to light. |

L001 four channel value description

| passageway | Channel value | function |
| --- | --- | --- |
| 1 | 000-255 | Total dimming, linear dimming, from dark to bright |
| 2 | 000-255 | RB linear dimming, adjusting from dark to light. |
| 3 | 000-255 | GW linear dimming, adjusting from dark to light. |
| 4 | 000-255 | Total stroboscopic, from slow to fast |

D001 two channel value description

| passageway | Channel value | function |
| --- | --- | --- |
| 1 | 000-255 | Total dimming, linear dimming, from dark to bright |
| 2 | 000-255 | Total stroboscopic, from slow to fast |

4、 Master slave setting

2Two or more identical lamps are connected by 3-core signal wires. If any host is set as voice control or gradual change, pulse change and jump change, and other lamps are in A001 mode, it will enter the master-slave mode and the slave display screen will not flash (flashing indicates that the machine has not received a signal, and the connecting wire must be checked)

Special attention: only one host can be set in a group of lamps. If there are multiple hosts, all lamps will flash randomly and step synchronously.