# MANUAL DEL USUARIO

Por  seguridad  y  una  mejor operación, por favor lea este manual cuidadosamente antes de usar y operar este equipo, para que no ocurra ninguna lesión personal ni daño a laluminaria.

Specifications

. Input Voltage: 110-240V AC, 50-60 Hertz

. Max Power Consumption: 360W

Light Source

. 24 x 15W LEDs(R,G,B,W)

Optical

. Lens Angle: 25° / 40°

Control Singal

. Protocol USITT DMX-512

. DMX Channels: 5/9

Cooling Method: Aluminum radiator

IP rate: IP20

Net Weight：4kgs

Dimension: 530×530×400mm / 4 pcs

Aviation box packaging

Menu

| passageway | Channel value | function |
| --- | --- | --- |
| 1 | 000-255 | Total dimming, linear dimming, from dark to bright |
| 2 | 000-255 | Total stroboscopic, from slow to fast |
| 3 | 0-29 | Dimming |
|  | 30-34 | Not bright |
|  | 35-39 | Light bead R is on |
|  | 40-44 | Light bead G is on |
|  | 45-49 | Light beads R and G are on |
|  | 50-54 | Light bead B is on |
|  | 55-59 | Light beads R and B are on |
|  | 60-64 | Light beads g and B are on |
|  | 65-69 | Light beads R, G, B, on |
|  | 70-74 | Light bead w on |
|  | 75-79 | Light beads R and W are on |
|  | 80-84 | Light beads g and W are on |
|  | 85-89 | Light beads R, G and W are on |
|  | 90-94 | Light beads B and W are on |
|  | 95-99 | Light beads R, B and W are on |
|  | 100-104 | Light beads g, B and W are on |
|  | 115-109 | Light beads R, G, B and W are on |
|  | 110-139 | Gradual change |
|  | 140-169 | Pulse change |
|  | 170-179 | Lamp bead R → g → B → w circulating light |
|  | 180-189 | Lamp bead w → B → G-R cycle lamp |
|  | 190-199 | Lamp beads R, G, and B, w cycle light |
|  | 200-209 | Lamp beads R, W and G, B cycle running lamp |
|  | 210-219 | Lamp beads R, B, and G, W, cycle lamp |
|  | 220-229 | Lamp beads R, G to g, B to B, w to W, R cycle light |
|  | 230-239 | Lamp beads R, G, B to g, B, w to B, W, R, cycle light |
|  | 240-249 | Lamp bead r-g-b-w circulation |
|  | 250-255 | Voice control |
| 4 | 000-255 | Channel 3 function speed, from slow to fast |
| 5 | 000-255 | R linear dimming, adjusting from dark to light. |
| 6 | 000-255 | G linear dimming, adjusting from dark to light. |
| 7 | 000-255 | B linear dimming, adjusting from dark to light. |
| 8 | 000-255 | W linear dimming, adjustable from dark to light. |